# RC Car

# Our project consisted in the creation of an RC racing car from scratch, and in fact the only things that we have bought are the electronics components such as the engine that were absolutely impossibile for us to make. Also we wanted to explore the field of aerodynamics and so we attached to the car a fan that would suck the air up from under the chassis thus creating a low pressure zone there therefore making the whole car more adherent to the ground.

## Supplies

## To achive greater speed we decided to use a brushless engine and for this purpouse we also had to use an ESC and a specific battery but one could just use a brushed (or DC) motor thus simplifying the electronics.

# Brushless engine (Hobbywing QuicRun Brushless Motor 3650 SD G2 - 13.5T)

# ESC (Hobby Wing QUICRUN 10BL60 Sensored)

# 4 tires (our scale is 1/10th thus the tires are of that size)

# A receiver and a controller (we bought a package whit both: ZHITING Turbo Racing 91803G-VT)

# 2 batteries (3S, 11V with a discharge of 50c and capacity of 2200 mAh, one for the motor, the other for the fan)

# 1 fan (brushless motor designed for cooling + fan blades by a generic computer fan)

# Wood pannels for the chassis

# 3D printer (for some components)

# 1 metal rod for the motor's axis

# 1 curved tube (where the fan was placed)

# Ball bearings

# Wires

# 1 servo motor

## Step 1: General Design

# We designed the car starting from the chassis, then we designed and modeled supports and connections for the wheels, the axis and the motor to print them with a 3D printer.

# The chassis was designed with holes to then attach all the supports.

## Step 2: Connecting the Electronics

# We soldered together the electronics following the diagram. We didn't want to connect two batteries to the same receiver thus we just connected the ground and the phase from the ESC of the fan so that it recognized the receiver while we connected everything from the motor's ESC. Also we followed the instructions from the manual of the receiver (you can find it on google if it isn't included in the box) to connect the right port.

## Step 3: Laser Cutting and 3D Printing

# At this point we proceeded to make the various parts of which we only had the 3D model. Thus we 3D printed the supports and the connectors and laser cut the chassis.

## Step 4: Making the Rear Axis

# Having everything we needed we started putting everything together. Thus we cut from a metal rod of 8mm of diameter the axis of the rear tires, drilled 3 holes into it so that we could fix the adapters of the tires and the connector of the motor. We bolted to it the connector of the motor, slided in the ball bearings so that they would fit in the supports and then bolted the tires' adapters.

## Step 5: Fixing the Supports

# We then fixed the supports to the chassis and fixed the axis between their top and bottom part and just screwed the rear tires to their adapters.