# ZEPP

## Description

Our project consisted of designing and building an airship that uses helium to regulate its altitude. The lift is provided by a mylar balloon filled with helium, which allows the airship to remain in air balance. To control movement during flight, we incorporated a set of propellers powered by a motor, enabling directional navigation and stability.

Attached to the balloon is a rigid platform that serves as the main structural base. This platform supports both the mechanical components such as the motor, propellers, and battery and the electronic system. The core of the electronic setup is an Arduino microcontroller, which manages the control signals and allows us to operate the airship remotely.

## Materials Used:

•	4 Mylar balloons

•	1 helium tank

•	Wooden base (platform)

•	Fishing lines (to tie the balloons)

•	2 ESP32 microcontrollers

•	Electronic wires

•	Hot glue

•	Small electric motors

•	Propellers

•	Remote controller

## Steps to follow to build the Airship

1.	Brainstorming and Planning

We started with a brainstorming session where we chose the materials to use and calculated the total weight of the airship. Using the basic idea of Archimedes’ principle, we estimated how much helium we would need to lift the structure.

2.	Collecting Materials and Assembling the Frame

Once we had all the necessary materials, we inflated four Mylar balloons and tied them with strings to a wooden platform. This platform would support both the mechanical and electronic parts of the airship.

3.	Installing the Side Propellers

First, we worked on the side propellers, which would allow the airship to move forward and backward. We connected them to the structure and made sure they were balanced and functional.

4.	Adding the Bottom Propeller

After putting together the main structure, we decided to add a third propeller underneath the platform. This new propeller was added to allow the airship to move up and down more easily.

5.	Programming the ESP32 and Arduino

We wrote the code for the ESP32 and Arduino to control the propellers. The code made it possible to activate the motors and change direction based on input signals.

6.	Remote Control Setup

To control the airship from a distance, we added a second ESP32 connected to a controller. This setup allowed us to send signals wirelessly to the airship and control the propellers remotely.

7.	Final Assembly

Once everything was ready, we connected all the electronic components to the wooden structure, including the motors, batteries, and microcontrollers. After checking all the connections, the airship was fully assembled.

8.	Testing and Adjustments

In the final step, we tested the airship to make sure everything worked correctly. We checked that the airship was air-balanced, meaning it could stay level in the air without tilting too much. We also verified that all the propellers worked properly, both for horizontal and vertical movement. After a few small adjustments, the airship was ready for use.